Quiz Questions for Lecture 1:

- What are the three biggest types of renewable energy? Why?

  - Wind power, solar power and hydropower. They are broadly distributed and in many cases close to population centers.

- Why is there more wind energy in mid-latitudes (35N to 60 N or 35S to 60S) than in other latitudes ( i.e. tropics or arctic)?

  - The differential heating between pole and equator creates frontal cyclones in these mid-latitude regions.

- How do frontal cyclones impact all three renewable energy types: wind, solar and hydro) ?

  - Frontal cyclones have strong winds that drive wind turbines. Their clouds and precipitation impact hydro and solar power

- Are the USA and China both in “mid-latitudes”?

  - Yes. Both lie mostly between 35N and 60N.

- Why does the renewable energy resource (wind, solar and hydro) vary with season? Which season has more renewable energy?

  - For wind, winter is better. There is more global differential heating in winter and thus more frontal cyclones.

  - For solar, summer is better. The sun angle is steeper in summer and there are fewer clouds

  - In most places, there is more precipitation in winter and less evaporation, so more hydropower. In a few places, summer snowmelt feeds the rivers.

- Why do locations with rising air in the atmosphere have more hydropower but less solar power?

  - Rising air in the atmosphere causes cloud formation and precipitation. Clouds block the sun’s rays reducing solar power. Precipitation provides water for hydropower.

- What are the four ways that mountains help create hydropower?

  - Mountains cause air to rise causing precipitation (i.e. orographic precipitation).

  - Mountains capture precipitation before it falls to sea level, giving the water potential energy.

  - Mountains channel the water into narrow rivers.

  - Mountain tops are cold, so they can store water as snow with little evaporation.

- What are two ways that mountains create more wind power?

  - Airflow accelerates over mountain tops.

  - Airflow accelerates in narrow mountain gaps.

- Why do offshore (i.e. ocean) locations have more wind power than forested landscapes or cities?

  - The surface roughness in forests and cities slows the wind by friction. Thee ocean surface is smoother, so the wind is stronger.